Assignment 6

Name: Suvikhyath Thirunahari
Roll No: 2503B05202
Stream: M tech CSE(Aiml)

Q1. LAB TASK: TCP Congestion Control Simulation

Question:
Using a TCP congestion control simulator (Python or any tool of your choice), simulate and analyze TCP Reno under three different packet-loss conditions:

- 0.1% loss

- 1% loss

- 2% loss

For each case:

- Plot the Congestion Window (cwnd) vs Time for at least 20 seconds of simulation.

- Compute the average throughput (in MSS/second or Mbps).

- Explain how packet loss affects the behavior of TCP Reno, specifically discussing:

  - Slow Start

  - Congestion Avoidance

  - Multiplicative Decrease (cwnd halving)

- Compare the three cases and write a short conclusion on how increasing loss impacts network performance.

import numpy as np

import matplotlib.pyplot as plt

def tcp_reno_sim(loss_rate, sim_time=20, rtt=0.05, mss=1):

"""

Improved TCP Reno simulation:

- loss checked per packet inside RTT

- clean Reno saw-tooth

"""

time_series = []

cwnd_series = []

cwnd = 1

ssthresh = 50

t = 0

while t < sim_time:

packets = int(cwnd)  # packets sent in this RTT

loss = False

# check packet loss inside RTT

for _ in range(packets):

if np.random.rand() < loss_rate:

loss = True

break

# record before cwnd update

time_series.append(t)

cwnd_series.append(cwnd)

if loss:

# Reno: multiplicative decrease

ssthresh = cwnd / 2

cwnd = max(1, cwnd / 2)

else:

# no loss

if cwnd < ssthresh:

cwnd += 1  # slow start

else:

cwnd += 1 / cwnd  # congestion avoidance

t += rtt  # move forward in time

# throughput in MSS per second

throughput = np.mean(cwnd_series) / rtt

return np.array(time_series), np.array(cwnd_series), throughput

# ---- Run 3 cases ----

loss_rates = [0.001, 0.01, 0.02]

labels = ["Loss 0.10%", "Loss 1.00%", "Loss 2.00%"]

colors = ["blue", "orange", "green"]

plt.figure(figsize=(12, 7))

for lr, lbl, c in zip(loss_rates, labels, colors):

t, cwnd, thr = tcp_reno_sim(lr)

plt.plot(t, cwnd, label=f"{lbl} (thr={thr:.2f} MSS/s)", color=c)

plt.title("TCP Reno Congestion Window vs Time")

plt.xlabel("Time (s)")

plt.ylabel("Congestion Window (MSS)")

plt.legend()

plt.grid(True)

plt.show()

output: